Geostress Inversion from Fracture Orientation Data

This document presents a complete, industry-grade methodology for estimating the full in-situ stress tensor from fracture azimuth and dip data. The workflow integrates deterministic mechanics, nonlinear inversion, Bayesian uncertainty analysis, stress polygons, DFIT/LOT calibration, and critically stressed fracture mapping. Applications include hydrocarbon reservoirs, geothermal systems, CCS, and hydrogen storage.

1. Introduction

Understanding the in-situ stress field is critical for fault stability, wellbore integrity, stimulation design, and subsurface storage safety. Fracture orientation data from image logs provides a powerful constraint on stress magnitudes and orientations when combined with frictional failure theory.

2. Stress and Fracture Geometry

2.1 Coordinate System

- x: East

- y: North

- z: Vertical (positive downward)

2.2 Fracture Plane Normal

For fracture azimuth αand dip δ:

n=[sin⁡δsin⁡α,sin⁡δcos⁡α,cos⁡δ]T

3. Stress Tensor and Traction

3.1 Principal Stress Tensor

σ=σH000σh000σv

3.2 Traction Vector

t=σn

3.3 Normal and Shear Stress

σn=nTσnτ=tTt-σn2

4. Failure Criterion

Assuming Mohr–Coulomb frictional sliding:

τ=μ(σn-Pp)

This equation forms the basis for inversion.

5. Nonlinear Inversion

5.1 System of Equations

Each fracture provides one nonlinear constraint. Multiple fractures yield an overdetermined system solved via least squares or Bayesian inversion.

5.2 Objective Function

F=i(τi-μ(σni-Pp))2

6. Bayesian Inference

6.1 Priors

- σH∼U(σv,2.5σv)

- σh∼U(0.6σv,1.5σv)

- μ∼N(0.6,0.05)

6.2 Outputs

- Posterior PDFs of stresses

- Confidence intervals

- Parameter trade-offs

7. Stress Polygon Constraints

Stress polygons define the allowable region of σH–σh space based on frictional faulting theory.

σ1-Ppσ3-Pp=(1+μ2+μ)2

Used to reject non-physical inversion results.

8. DFIT / LOT Calibration

- σh≈PISIP

- σvfrom density logs

- σHbounded by loss circulation and breakouts

9. Critically Stressed Fractures

9.1 Slip Tendency

Ts=τ/(σn-Pp)

9.2 Dilation Tendency

Td=(σ1-σn)/(σ1-σ3)

10. Figures to Include

- Stress polygon (σH vs σh)

- Mohr circles with fracture planes

- SHmax rose diagram

- Slip tendency stereonet

11. Example Field Dataset (Synthetic)

- Depth: 3200 m

- σv=75MPa

- Pp=35MPa

- DFIT: σh=52MPa

- SHmax azimuth: 060°

Inversion result:

- σH=92±6MPa

- σh=52±3MPa

12. Applications

- Hydraulic fracturing design

- Fault reactivation risk

- CCS and H₂ storage integrity

- Geothermal reservoir optimization

13. Conclusions

Fracture-based geostress inversion, when integrated with independent constraints and uncertainty analysis, provides a robust and defensible estimate of the in-situ stress field suitable for operational decision-making and publication.

Formulation for estimating in-situ geostress components from fracture orientation (azimuth & dip).

This Is Used For:

✔ Stress magnitude estimation
✔ SHmax orientation validation
✔ Critically stressed fracture prediction
✔ Fault reactivation risk
✔ Reservoir geomechanics & wellbore stability

Integrated Workflow

- Density → σv

- DFIT → σh

- Image logs → fracture azimuth/dip

- Bayesian inversion → σH,σhPDFs

- Stress polygon filtering

- Critical stress mapping

- Well & stimulation optimization

1. Geometry of the fracture plane

Let:

- Azimuth of fracture strike = α(measured clockwise from North)

- Dip of fracture plane = δ

- Coordinate system:

  - x= East

  - y= North

  - z= Vertical (positive downward)

Unit normal vector to the fracture plane

n=sin⁡δ sin⁡αsin⁡δ cos⁡αcosδ

2. In-situ stress tensor

Assuming principal stresses aligned with geographic axes:

σ=σH000σh000σv

Where:

- σH= maximum horizontal stress

- σh= minimum horizontal stress

- σv= vertical stress

3. Traction vector on the fracture plane

t=σ⋅n
t=σHsin⁡δsin⁡ασhsin⁡δcos⁡ασvcos⁡δ

4. Normal stress on the fracture

σn=n⋅t
σn=σH(sin⁡δsin⁡α)2+σh(sin⁡δcos⁡α)2+σv(cos⁡δ)2

5. Shear stress on the fracture

Magnitude of shear stress:

τ=t⋅t-σn2

Expanded:

τ=σHsin⁡δsin⁡α)2+(σhsin⁡δcos⁡α)2+(σvcos⁡δ)2-σn2

6. Fracture failure condition (Mohr–Coulomb)

If the fracture is critically stressed:

τ=μ(σn-Pp)

Where:

- μ= friction coefficient (≈ 0.6 for crustal rocks)

- Pp= pore pressure

7. Solving for geostress

Given:

- Measured α,δ

- Known or estimated σv

- Assumed μ, Pp

You can solve the system:

σn(σH,σh)τ(σH,σh)⇒σH, 						σh

Using:

- Multiple fracture planes

- Least-squares or inversion approach

A) Matrix / Inversion Formulation

A1. Stress tensor (unknowns)

Assume Andersonian regime:

σ=σH000σh000σv

Unknowns:

x=σHσh(σv known from overburden)

A2. Fracture normal vector

Given:

- Azimuth α

- Dip δ

n=nxnynz=sin⁡δsin⁡αsin⁡δcos⁡αcosδ

A3. Normal stress in linear form

σn=σHnx2+σhny2+σvnz2

Rearranged:

σn-σvnz2=nx2ny2σHσh

A4. Shear stress expression

Traction magnitude:

∣t∣2=σH2nx2+σh2ny2+σv2nz2
τ2=∣t∣2-σn2

A5. Mohr–Coulomb failure constraint

τ2=μ2(σn-Pp)2

This produces one nonlinear equation per fracture.

A6. Inversion system (multiple fractures)

For Nfractures:

F(σH,σh)=τ12-μ2(σn1-Pp)2τ22-μ2(σn2-Pp)2⋮τN2-μ2(σnN-Pp)2=0

Solved via:

- Least squares

- Nonlinear optimization

- Bayesian inversion (preferred)

B) Worked Numerical Example

Given:

- α=045∘

- δ=60∘

- σv=50MPa

- Pp=20MPa

- μ=0.6

Step 1: Unit normal

nx=sin⁡60∘sin⁡45∘=0.612
ny=sin⁡60∘cos⁡45∘=0.612
nz=cos⁡60∘=0.5

Step 2: Normal stress

σn=0.375σH+0.375σh+12.5

Step 3: Solve assuming critical stress

Try:

σH=80 MPa,σh=60 MPa
σn=65 MPa
τ=0.6(65-20)=27 MPa

✔ This fracture is critically stressed

E) Extension: Rotated Stress Tensor

If SHmax azimuth θ≠geographic axes:

Rotation matrix (about vertical axis)

R=cosθ-sin⁡θ0sinθcosθ0001

Global stress tensor

σglobal=RσprincipalRT

Then use:

t=σglobaln

Same equations apply afterward.

Bayesian Inversion with Uncertainty Bounds

Instead of a single solution for σH,σh, we estimate probability distributions.

1.1 Model definition

Unknowns:

x=[σH,σh]

Forward model (for each fracture i):

gi(x)=τi-μ(σni-Pp)

Assume Gaussian observational error:

p(d∣x)∝exp⁡12igi2σi2

1.2 Prior distributions

Typical realistic priors:

σH∼U(1.0σv,2.5σv)
σh∼U(0.6σv,1.5σv)
μ∼N(0.6,0.05)

1.3 Bayesian output

✔ Mean stress magnitudes
✔ 95% confidence intervals
✔ Trade-off between σHand σh
✔ Fracture-by-fracture misfit

2️⃣ Stress Polygon Construction (Zoback Method)

Stress polygons constrain allowable σH–σh pairs.

2.1 Stress regimes

Using Andersonian theory:

- Normal faulting:

σv>σH>σh

- Strike-slip faulting (most reservoirs):

σH>σv>σh

- Reverse faulting:

σH>σh>σv

2.2 Frictional fault limits

For optimally oriented faults:

σ1-Ppσ3-Pp=1+μ2μ2

This gives upper and lower bounds on σHand σh.

2.3 Polygon usage

✔ Eliminate non-physical solutions
✔ Combine with fracture inversion
✔ Validate with borehole breakouts & drilling losses

3️⃣ DFIT / LOT / Mini-frac Calibration

Fracture-based inversion alone is non-unique. Calibration is essential.

3.1 Minimum horizontal stress

From DFIT / LOT:

σh≈PISIP

Where:

- PISIP= instantaneous shut-in pressure

3.2 Vertical stress

σv(z)=0zρ(z)g dz

Use:

- Density logs

- Checkshot-calibrated depth

3.3 Maximum horizontal stress

σH=σh+μ(σv-Pp)1-μ(strike-slip regime)

Used as hard constraint in inversion.

3.4 Integrated workflow

| Data | Constraint |
| --- | --- |
| DFIT | σh |
| Density log | σv |
| Fractures | Stress orientation & ratios |
| Breakouts | SHmax azimuth |
| Losses | Upper bound σH |

4️⃣ Critically Stressed Fracture Mapping

Once stress tensor is known:

4.1 Slip tendency

Ts=τσn-Pp

- Ts≥μ→ slipping

- Ts≈0.8μ→ critically stressed

4.2 Dilation tendency

Td=σ1-σnσ1-σ3

Used for:
✔ Natural fracture permeability
✔ Stimulation sweet spots
✔ CO₂ / H₂ storage risk

5️⃣ Regional Stress Logic (Egypt / Middle East)

General tectonic context:

- Dominant strike-slip to transtensional regime

- SHmax often:

  - ENE–WSW (Gulf of Suez / Nile Delta)

  - NE–SW (Eastern Mediterranean)

Implications:
✔ Reactivation-prone NE–SW faults
✔ Vertical fractures subparallel to SHmax
✔ Higher stimulation efficiency along SHmax-aligned wells

Geostrain Equations

Geostrain represents in-situ deformation due to tectonics, compaction, pore-pressure change, and temperature.
In reservoir geomechanics, stresses are computed from strains using poroelastic constitutive laws.

  Stress from fractures

  Stress from strain (poroelasticity)

3A.1 Total Strain Tensor

ε=εxxεxyεxzεyxεyyεyzεzxεzyεzz

3A.2 Linear Poroelastic Stress–Strain Law (Biot)

σ=C:(ε-εT)-αPpI

Where:

- C= elastic stiffness tensor

- εT= tectonic strain tensor

- α= Biot coefficient (0.6–1.0)

- Pp= pore pressure

3A.3 Isotropic Poroelastic Form (Most Oil & Gas Use)

σij=2G(εij-εijT)+λ(εv-εvT)δij-αPpδij

With:

G=E2(1+ν),λ=Eν1+ν)(1-2ν
εv=εxx+εyy+εzz

3A.4 Principal Stress–Strain Relations

ε1=1Eσ1-ν(σ2+σ3)+αPpE+ε1T
ε2=1Eσ2-ν(σ1+σ3)+αPpE+ε2T
ε3=1Eσ3-ν(σ1+σ2)+αPpE+ε3T

3A.5 Horizontal Stress from Measured Geostrain

σh=E1-ν2(εh+νεH)+αPp
σH=E1-ν2(εH+νεh)+αPp

Used when horizontal strains are known or inferred (image logs, subsidence).

3A.6 Uniaxial Strain Model (Classic Reservoir Compaction)

Assumptions:

εx=εy=0
σh=σH=ν1-νσv+1-2ν1-ναPp

This is the most-used stress model for depletion forecasting.

3A.7 Stress Path Coefficient

K=ΔσhΔPp=α1-2ν1-ν

Typical values:

- Sandstone: 0.6–0.8

- Carbonate: 0.7–0.9

- Shale: 0.8–1.0

3A.8 Thermal Strain (Injection / Geothermal / CCS)

εT=αTΔT
ΔσT=EαTΔT1-ν

Critical for:

- Water injection

- CO₂ storage

- Geothermal cooling

3A.9 Coupled Stress–Strain–Fracture Workflow

- Compute geostrain (tectonic + compaction)

- Convert strain → stress (poroelasticity)

- Rotate stress tensor

- Apply fracture inversion

- Evaluate fault reactivation

3A.10 Applications

✔ Reservoir depletion & subsidence
✔ Sanding & compaction prediction
✔ Injection-induced seismicity
✔ Geothermal / CCS integrity
✔ Wellbore stability & stimulation

Example Field Dataset (Synthetic)

- Depth: 3200 m

- σv=75MPa

- Pp=35MPa

- DFIT: σh=52MPa

- SHmax azimuth: 060°

Inversion result:

- σH=92±6MPa

- σh=52±3MPa

- Geothermal / CCS / H₂ Modifications

- Include thermal stress term: ΔσT = EαΔT/(1−ν)

- Long-term pore pressure evolution

- Fault permeability evolution

- Induced seismicity screening

Geostrain FormulationBottom of Form

Geostrain analysis complements geostress by describing rock deformation under in-situ loading, pore pressure, and temperature changes. This section provides industry-standard strain equations used in reservoir geomechanics, compaction, subsidence, hydraulic fracturing, and depletion studies.

21.1 Strain Tensor Definition

The infinitesimal strain tensor is defined as:

ε = [ε_ij] = 1/2 (∂u_i/∂x_j + ∂u_j/∂x_i)

where u is the displacement vector.

Normal strains:

- ε_xx = ∂u_x/∂x

- ε_yy = ∂u_y/∂y

- ε_zz = ∂u_z/∂z

Shear strains:

- γ_xy = 2ε_xy

- γ_xz = 2ε_xz

- γ_yz = 2ε_yz

21.2 Linear Elastic Constitutive Law (Hooke’s Law)

For isotropic linear elasticity:

ε_xx = (1/E)[σ_xx − ν(σ_yy + σ_zz)] ε_yy = (1/E)[σ_yy − ν(σ_xx + σ_zz)] ε_zz = (1/E)[σ_zz − ν(σ_xx + σ_yy)]

Shear strains: γ_xy = τ_xy / G γ_xz = τ_xz / G γ_yz = τ_yz / G

with shear modulus: G = E / [2(1+ν)]

21.3 Principal Stress–Strain Relationship

In principal coordinates:

ε_1 = (1/E)[σ_1 − ν(σ_2 + σ_3)] ε_2 = (1/E)[σ_2 − ν(σ_1 + σ_3)] ε_3 = (1/E)[σ_3 − ν(σ_1 + σ_2)]

Volumetric strain:

ε_v = ε_1 + ε_2 + ε_3 = (1−2ν)/E (σ_1 + σ_2 + σ_3)

21.4 Poroelastic Strain (Biot Theory)

Effective stress principle:

σ'_ij = σ_ij − α P_p δ_ij

Resulting strain:

ε_v = (1/K)(σ_m − αP_p)

where:

- σ_m = (σ_1 + σ_2 + σ_3)/3 (mean stress)

- α = Biot coefficient (0.6–1.0)

- K = bulk modulus

Reservoir depletion strain:

Δε_v = − α ΔP_p / K

This governs compaction and surface subsidence.

21.5 Uniaxial Strain Condition (Reservoir Compaction)

Common oil & gas assumption (ε_x = ε_y = 0):

Δσ_h = ν/(1−ν) Δσ_v + α/(1−ν) ΔP_p

Used for:

- Depletion-induced stress changes

- Time-lapse geomechanics

- Fracture reorientation prediction

21.6 Thermal Strain

Thermal strain due to temperature change:

ε_T = α_T ΔT

Thermoelastic stress (constrained rock):

Δσ_T = E α_T ΔT / (1 − ν)

Critical for:

- Steam injection

- Geothermal wells

- H₂ and CO₂ injection

21.7 Strain from Pore Pressure Change (Reservoir Depletion)

Vertical strain:

ε_z = (1/E)[Δσ_v − ν(Δσ_H + Δσ_h)] − αΔP_p/E

Horizontal strain:

ε_h = (1/E)[Δσ_h − ν(Δσ_H + Δσ_v)] − αΔP_p/E

21.8 Practical Oil & Gas Applications

- Reservoir compaction & subsidence prediction

- Stress path modeling (K₀ path)

- Fracture conductivity evolution

- Fault reactivation screening

- Time-lapse (4D) geomechanics

21.9 Recommended Data Inputs

- Static & dynamic Young’s modulus

- Poisson’s ratio from logs & lab

- Biot coefficient

- Pore pressure history

- Temperature history

- DFIT stress calibration

21.10 Deliverables

Always integrate geostrain modeling with fracture-based geostress inversion. Stress defines failure; strain defines deformation. Together, they provide a complete and defensible geomechanical framework for oil, gas, geothermal, CCS, and subsurface storage projects.

Geostrain Formulation for Oil & Gas Applications” now includes:

- Full strain tensor definitions (normal + shear)

- Hooke’s law in both Cartesian and principal stress space

- Volumetric strain and compaction equations

- Poroelastic (Biot) strain formulation used in reservoir depletion

- Uniaxial strain (K₀ path) equations — industry standard

- Thermal strain and thermoelastic stress (critical for geothermal, steam, CCS, H₂)

- Stress–strain coupling for pore pressure changes

- Clear oil & gas operational use cases

- Recommended input data (logs, lab, DFIT, pressure, temperature)

- A strong industry recommendation section tying stress + strain together